# ASTRALINK FSOC Prototype: Mathematical Formulas and Calculations

This document outlines the core mathematical models, calculations, and algorithms powering the ASTRALINK Virtual Camera Tracking System (FSOC prototype).

# 1. Kalman Filter Tracking Model

The beacon tracker employs a constant-velocity Kalman filter to estimate the position and velocity of the target across frames, bridging temporary signal losses and predicting future positions.

## 1.1 State Vector

The state vector x contains the 2D position (x, y) and 2D velocity (v_x, v_y) of the beacon in the image plane.

x = [x, y, v_x, v_y]^T

## 1.2 Prediction Step

The state transition model assumes constant velocity over a time step dt (nominal 1 / UPDATE_RATE).

F (State Transition Matrix):

F = [[1, 0, dt, 0],
     [0, 1, 0, dt],
     [0, 0, 1, 0],
     [0, 0, 0, 1]]

Predicted State: x_pred = F * x_prev

Process Noise Covariance (Q):

Q = accel_var * [[dt^4/4, 0, dt^3/2, 0],
                 [0, dt^4/4, 0, dt^3/2],
                 [dt^3/2, 0, dt^2, 0],
                 [0, dt^3/2, 0, dt^2]]

Predicted Covariance: P_pred = F * P_prev * F^T + Q

## 1.3 Measurement Update

The measurement vector z contains the detected beacon position (x_m, y_m).

Observation Matrix (H):

H = [[1, 0, 0, 0],
     [0, 1, 0, 0]]

Innovation (Residual): y = z - H * x_pred

Innovation Covariance (S): S = H * P_pred * H^T + R (where R is measurement noise)

Kalman Gain (K): K = P_pred * H^T * S^-1

Updated State: x_new = x_pred + K * y

Updated Covariance: P_new = (I - K * H) * P_pred * (I - K * H)^T + K * R * K^T

## 1.4 Confidence & Uncertainty

Position Uncertainty = sqrt(P[0,0] + P[1,1])

Confidence Score = 1.0 / (1.0 + Position Uncertainty)

# 2. Camera Kinematics and Control

The system calculates angular pan and tilt commands based on pixel offset errors.

## 2.1 Field of View Mapping

Pixels per degree mapping:

Px = FRAME_WIDTH / FOV_HORIZONTAL

Py = FRAME_HEIGHT / FOV_VERTICAL

## 2.2 Control Law

Target angle corrections are computed from the offset between the predicted state and the camera center:

Delta Pan = (x_pred - center_x) / Px

Delta Tilt = (y_pred - center_y) / Py

The physical controller smooths and applies these corrections as bounded velocity inputs to the simulation.

# 3. Environmental Disturbances

The simulator introduces mathematical models for atmospheric and physical platform disturbances.

## 3.1 Camera Jitter

A correlated random walk models high-frequency mechanical vibration:

J_offset(t) = 0.72 * J_offset(t-1) + 0.28 * N(0, 0.35 * max_jitter)

## 3.2 Atmospheric Turbulence (Beam Wander)

Low-frequency spatial remapping is generated via Gaussian-blurred independent noise fields for X and Y:

dx_field = GaussianBlur(N(0,1))

dy_field = GaussianBlur(N(0,1))

The fields are normalized such that the maximum displacement equals the configured turbulence strength, then applied using inverse remapping.

## 3.3 Atmospheric Veil (Haze/Fog)

Contrast reduction and haze are modeled via alpha blending with a solid or spatially-varying veil color:

Output_pixel = Input_pixel * (1.0 - alpha) + Veil_color * alpha

# 4. Target Motion Models

Trajectories are driven by trigonometric or linear parametric equations over frame count (t).

## 4.1 Circular Motion

x(t) = center_x + R_x * cos(w * t + phase)

y(t) = center_y + R_y * sin(w * t + phase)

## 4.2 Figure-8 Motion

x(t) = center_x + R_x * sin(w * t + phase)

y(t) = center_y + R_y * sin(w * t + phase) * cos(w * t + phase)

# 5. Performance Metrics

Real-time tracking accuracy is quantified using error and retention statistics.

## 5.1 Tracking Error

Euclidean pixel distance between estimated track and ground truth:

Error(t) = sqrt((x_est - x_true)^2 + (y_est - y_true)^2)

## 5.2 Root Mean Square Error (RMSE)

RMSE = sqrt( sum(Error_i^2) / N )

## 5.3 Target Loss Rate

Percentage of active evaluation frames where no valid detection was matched:

Loss % = (Missed Frames / Total Evaluation Frames) * 100

## 5.4 Re-acquisition Time

Time span between the onset of signal loss and the restoration of a TRACKING lock:

T_reacq = (Frame_recovered - Frame_lost) / UPDATE_RATE (seconds)